Project report

Name – Garvit Agarwal

CTAE College , Udaipur

B.tech (Electronics and Communication Engineering ) 2th year

Project – 1 (energy meter )

- Aim - To design and build an energy meter using Arduino to measure voltage, current, and power consumption.

Components required

- Arduino Uno

- LCD (16x2, I2C)

- Current Sensor (ACS712)

- Transformer

- Diodes

- Resistors, Capacitors

- DC Motor (if used)

Working

- Voltage is stepped down through the transformer

- Then converted to the DC using rectifier

- Current is measured using sensor

- Arduino processes data

- Output shown on LCD

Schematic Diagram

PCB Layout

Main Coding concepts

- Current values and voltage values are taken as input

- The they are converted into their real values

- Power is calculated using current and voltage

- Time is measured using millis() function

- Energy is calculated using power and time

- Then both the values are shown on the lcd display

THE testing and building are ongoing

Project – 2 (DC voltmeter)

Aim – To design and build the dc voltmeter using Arduino that measures the voltage of a dc voltage source or battery

Components Required –

- Battery (for testing)

- Breadboard

- Jumper wires

- Arduino uno r3

- 2 resistors – 1 k ohm and 220 ohms

Working

- The DC voltage is given to the voltage divider circuit

- The voltage divider converts it with in the safe range of the Arduino (5v)

- Arduino reads the analog voltage using its ADC (Analog-to-Digital Converter).

- The analog value is converted into actual voltage using a formula.

- The calculated voltage is displayed on the LCD or Serial Monitor.

Schematic Diagram

PCB Layout

Main Coding Concepts

- The value of the resistors given

- Calibration factor is given (in this the multimeter is not used instead considering the previous results the calibration factor is determined )

- Serial monitor is initialized

- The voltage form the divider circuit is taken as an input

- Converts the analog data into the voltage

- Applying the reverse voltage divider , converting back the safe range voltage into its original range

- Calibration factor is multiplied

- And finally the result is printed on the serial monitor

RESULT

The DC voltmeter was successfully implemented using Arduino, demonstrating accurate voltage measurement and display using a simple and cost-effective setup.

Project – 3(Temperature monitoring system)

Aim – To design and build the temperature monitoring system using Arduino and the dht / LM35 sensor and display real time temperature

Components required –

- Arduino Uno

- Temperature Sensor (LM35 / DHT11)

- Breadboard / PCB

- Connecting Wires

Working –

- The temperature sensor (LM35/DHT11) detects ambient temperature.

- LM35 outputs an analog voltage proportional to temperature.

- Arduino reads this value using an analog pin.

- The value is converted into temperature (°C).

- The result is displayed on LCD or Serial Monitor.

Schematic Diagram

PCB Layout –

Main Coding Concepts

- The input from the potentiometer and the temperature sensor is taken

- Serial monitor is initialized

- The range of the temperature analog input and the potentiometer analog input is set

- Placing the if else condition

- If the sensor temperature is more than the potentiometer limit then sent alert , other wise keep displaying the temperature

Result –

The project successfully measures and displays temperature using Arduino and a temperature sensor, demonstrating a simple and effective monitoring system.

Project – 4 (Metal Detector)

Aim – To design and implement a metal detector using Arduino to detect the presence of metallic objects.

Components Required –

- Arduino Uno

- Coil (Inductor)

- Capacitor

- Buzzer

- Resistors

- Breadboard / PCB

- Connecting Wires

Working –

- A coil is used to generate a magnetic field when current flows through it.

- When a metal object comes near the coil, it disturbs the magnetic field.

- This change affects the circuit parameters (inductance/frequency).

- Arduino detects this variation and triggers a buzzer as an alert.

Schematic Diagram –

PCB Layout –

Main Coding Concepts –

- Arduino monitors the signal from the detection circuit

- Detects change in frequency or voltage

- If change exceeds threshold → activates buzzer

Result –

The project successfully demonstrates detection of metal objects using electromagnetic principles and Arduino, providing a simple and effective detection system.